POBCO Performance Materials

POBCO-B® HARDWOOD (NO-STATIC BUILDUP) POBCO-B lubricant impregnated

hardwood bearings, wearstrips and components often

provide the best overall service of our various bearing materials. These

natural fiber bearings are 100% filled with a blend of fluid lubricants.

They render a greater speed/ load capability and have a lower wear

rate factor than many plastics. This material is suitable for indirect

contact only in the food and beverage industry. (100% Dielectric)

POBCO UHMW (RATED FOR 180 DEG F) AND POBCO HI-TEMP UHMW

(RATED FOR 275 DEG F) Both the UHMW and Hi-Temp UHMW

bearings and wear strips are made from Ultra High Molecular Weight

Polyethylene (UHMW). This material is widely used to reduce friction

from metal on metal contact. Applications are often in the presence

of abrasive particles, wet environments, and will tolerate exposure to

many chemicals. It is FDA and USDA approved for direct contact with

food products. (special blends and colors available)

POBCO PLUS® POBCO Plus bearings and components are manufactured

utilizing a food grade fluid lubricant united with UHMW plastic. This

product was developed to retain the basic properties of UHMW while

creating a plastic bearing material which reduces the coefficient of

friction and wear rate factor. POBCO Plus meets all official requirements

for use in the food and beverage industry. Reduces drag and friction up

to 40%.

POBCO BLUE UXC UHMW is primarily used as Guide material for Table

Top and Roller Chain applications. This blend of UHMW has a coefficient

of friction of 50% to 75% less than standard UHMW when used with

Acetal, PET and Steel Table Top Chain. Other characteristics include:

outstanding dry-running properties, FDA compliant, noise reducing, and

eliminates the slip-stick effect (back-sliding).

POBCO BLUE “AW” is our lowest coefficient of friction formula.

Lubricant is uniformly dispersed for continued lubricity throughout

the cross section. Reduces tendency to “stick-slip” under intermittent

dynamic loads. Ideal for belt guides, roller chain guides, and table top

applications and is FDA compliant. Color: Royal blue

POBCO STATIC-DISSIPATIVE /ANTI-STATIC MATERIALS POBCO offers several

plastics materials with varying degrees of static-reduction capabilities,

including black UHMW. These materials can help solve electrical

discharge and dirt/ grime attraction problems. These materials offer

excellent wear and bearing properties and are available in many

standard and custom shapes and sizes including Guide Rails, Wear

Strips, Clip-Ons, Bearings and Conveyor Rollers. Minimums may apply.

These plastic materials are not FDA/USDA compliant.

Static-Dissipative Range: Surface Resistivity 105 - 109 ohms/square Anti-

Static Range: Surface Resistivity 109 - 1014 ohms/square

POBCO HI-WEAR UHMW 33% more wear resistant than unfilled UHMW.

Excellent for use as Chain Guides, Bottle-Neck Guides, Liners and in

Waste Handling applications. Not FDA/USDA compliant.

ACETAL (DELRIN®) POBCO molded Delrin bearings and components

find their primary usage in light to medium duty applications. POBCO

machined Delrin bearings can be made for heavier duty applications.

Our Delrin equipped rollers have low overall cost while affording long

service life. Acetal is FDA, USDA compliant. Acetal is more resistant to

hot steam cleaning than UHMW.

POBCO NYWEAR (FDA & NON FDA MATERIALS) POBCO Nywear bearings

and components are especially suited for high heat (230oF+) material

handling applications. This fluid lubricant-filled plastic renders

exceptional wear and abrasive resistant capabilities with a low heat

build up. High PV limit.

POBCO NYLON MD (NYLATRON GS®) POBCO Dry Lubricant-Filled Nylon

bearings and components are preferred for heavy load, high speed

operations up to approximately 220oF operating temperatures. Color is

charcoal gray/black.

RULON® LR (NON FDA) Specially compounded forms of PTFE

Fluropolymer used as a bearing material which requires no lubrication

and is capable of performing under severe temperature and corrosive

conditions. The operating temperature ranges from - 400o to +550oF.

RULON® 641 white plastic is approved for use in the Food, Drug, and

Chemical Industry. This material is common for high heat applications

up to 550oF. It’s friction decreases rapidly with increasing load. Other

grades of Rulon also are available with higher/lower PV values for

nonfood applications.

POBCO PTFES Highly chemically resistant, with operating temp. range of

450o - 550oF depending on the additive. High PV ratings and some food

grades available.

POBCO POBLON is a Hi-Temp (500oF), proprietary blend of PTFE.

POBLON is FDA/USDA compliant. Used in food & pharmaceutical

applications such as ovens, fryers, proofing & curing operations. A less

expensive alternative to Rulon 641. Color is off-white.

PEEK® High Temp. 480oF, High PV limits, low coefficient of friction

excellent chemical & wear resistant.

ERTALYTE® PET-P (POLYESTER) Excellent dimensional stability, less

moisture absorption & better wear resistance than many plastics,

operates at 210oF continuously, FDA compliant, weather resistant. Good

chemical and abrasion resistance.

POBCO DRY WEAR DRY LUBE FILLED UHMW Black color Non-FDA.

Improved coefficient of friction and anti-static properties. Better in

dusty applications: less build-up. UV stabilized.

Metal detectable Acetal and UHMW now

available. Please inquire for more details.

It is the responsibility of the user to test and

approve the metal detection of this product due

to variations in metal detection systems and

required sensitivity settings.